College of Computer Studies

Laboratory Exercise No. 5

| Topic: | Temperature and Humidity Sensor in ESP32 | Week No. | 2 |
| --- | --- | --- | --- |
| Course Code: | CSST105 | Term: | Inter Semester |
| Course Title: | Robotics | Academic Year: | 2025-2026 |

Temperature and Humidity Sensor in ESP32

Objective:

To design a system using the ESP32 to collect and transmit temperature and humidity data from a DHT11/22 sensor.

At the end of this laboratory exercise, the students should be able to:

- Interface a DHT11/DHT22 sensor with the ESP32 microcontroller.

- Acquire and process temperature and humidity data.

- Implement conditional decision-making algorithms.

- Design an environmental alert system using LEDs and a buzzer.

- Analyze collected sensor data and evaluate system performance.

Materials Required:

- ESP32 Development Board

- DHT11/DHT22 Temperature and Humidity Sensor

- Breadboard and Jumper Wires

- Buzzer, LEDs, and pull-up resistors

- Arduino IDE

Pre-Lab Preparation:

- Software Installation:

  - Install the Arduino IDE and set it up for ESP32 development.

- Wiring Setup:

Identify DHT Sensor Pins:

DHT sensors typically have three pins:

- VCC (Power)

- GND (Ground)

- Data (Signal)

Wiring the DHT Sensor:

VCC Pin:

Connect the VCC pin of the DHT sensor to the 3.3V pin on the ESP32.

GND Pin:

Connect the GND pin of the DHT sensor to a GND pin on the ESP32.

Data Pin:

Connect the Data pin of the DHT sensor to a GPIO pin on the ESP32 (e.g., GPIO 23).

Adding a Pull-Up Resistor (Optional but Recommended):

Connect a 4.7kΩ to 10kΩ resistor between the Data pin and VCC. This helps stabilize the signal.

Double-Check Connections:

Ensure all connections are secure and correctly placed according to the pins defined.

- Download and Install Bluetooth Serial Terminal on Google Playstore (for android only)

Lab Activity Steps:

- Set Up the ESP32 Environment:

  - Open Arduino IDE, select the ESP32 board, and choose the appropriate COM port.

- Code Development:

  - Create a new sketch and include the necessary libraries:

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <DHT.h>

// LCD setup (I2C address 0x27, 16 columns, 2 rows)

LiquidCrystal_I2C lcd(0x27, 16, 2);

// DHT11 sensor setup

#define DHTPIN 23     // GPIO pin connected to DHT11 data pin

#define DHTTYPE DHT11 // DHT 11 sensor

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

lcd.init();       // Initialize LCD

lcd.backlight();  // Turn on backlight

dht.begin();      // Initialize DHT sensor

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Starting...");

delay(1000);

}

void loop() {

float t = dht.readTemperature();

float h = dht.readHumidity();

// Check if any reads failed and exit early (to try again).

if (isnan(t) || isnan(h)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("DHT11: ");

Serial.print(t);

Serial.print(" C, ");

Serial.print(h);

Serial.println(" %");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print((int)t);

lcd.print(" C");

lcd.setCursor(0, 1);

lcd.print("Humidity: ");

lcd.print((int)h);

lcd.print(" %");

delay(5000); // Wait 5 seconds before next reading

}

- Make the circuit

- Upload the Code:

  - Connect the ESP32 and upload the code.

- Data Monitoring:

  - Observe real-time temperature and humidity readings in the LCD Display.

Experimental Procedure:

Part A: Sensor Interfacing

- Construct the circuit according to the wiring table.

- Install the DHT sensor library.

- Verify that temperature and humidity values are displayed in the Serial Monitor. \

Part B: Environmental Classification

Program the ESP32 to classify the environment into:

Normal

- Green LED ON

- Red LED OFF

- Buzzer OFF

Warning

- Green LED OFF

- Red LED BLINKING

- Buzzer beeps every 2 seconds

Critical

- Green LED OFF

- Red LED FAST BLINKING

- Continuous buzzer alarm

Environmental Condition Classification Table

| Status | Temperature (°C) | Humidity (%) | System Response |
| --- | --- | --- | --- |
| Normal | 18°C – 20°C | 40% – 60% | Green LED ON, Buzzer OFF |
| Warning | 21°C – 25°C | 30% – 39% OR 61% – 75% | Yellow LED ON (or Red LED Blinking), Slow Buzzer |
| Critical | ≥ 30°C | < 30% OR > 75% | Red LED ON, Rapid Buzzer Alarm |

### Alternative Decision Matrix

| Temperature Range | Humidity Range | Status |
| --- | --- | --- |
| 18°C – 20°C | 40% – 60% | Normal |
| 21°C – 25°C | 30% – 39% | Warning |
| 21°C – 25°C | 61% – 75% | Warning |
| ≥ 30°C | < 30% | Critical |
| ≥ 30°C | > 75% | Critical |

## Example Readings

| Temperature (°C) | Humidity (%) | Status |
| --- | --- | --- |
| 19 | 50 | Normal |
| 20 | 45 | Normal |
| 23 | 35 | Warning |
| 25 | 70 | Warning |
| 30 | 28 | Critical |
| 32 | 80 | Critical |
| 35 | 85 | Critical |

Suggested Logic for ESP32

#include <DHT.h>

#include <LiquidCrystal_I2C.h>

#define DHTPIN 4

#define DHTTYPE DHT22		#DHT11 if no DHT22

#define GREEN_LED 18

#define RED_LED 19

#define BUZZER 23

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

pinMode(GREEN_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

dht.begin();

}

void loop() {

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Sensor Error!");

return;

}

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.print(" °C   ");

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

if ((temperature >= 18 && temperature <= 20) &&

(humidity >= 40 && humidity <= 60))

{

status = "NORMAL";

}

else if ((temperature > 20 && temperature <= 25) ||

(humidity >= 30 && humidity < 40) ||

(humidity > 60 && humidity <= 75))

{

status = "WARNING";

}

else if (temperature >= 30 ||

humidity < 30 ||

humidity > 75)

{

status = "CRITICAL";

}  else if (temperature > 30 || humidity < 40) {

digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED, HIGH);

delay(250);

digitalWrite(RED_LED, LOW);

digitalWrite(BUZZER, HIGH);

delay(250);

digitalWrite(BUZZER, LOW);

Serial.println("STATUS: WARNING");

}

else {

digitalWrite(GREEN_LED, HIGH);

digitalWrite(RED_LED, LOW);

digitalWrite(BUZZER, LOW);

Serial.println("STATUS: NORMAL");

}

delay(2000);

}

Guide Questions

- What is the difference between DHT11 and DHT22 in terms of accuracy and range?

- Why is environmental monitoring important in robotic systems?

- How does an ESP32 process digital sensor data?

- What is the purpose of implementing warning and critical thresholds?

- How can IoT improve environmental monitoring systems?

- What challenges were encountered during sensor interfacing?

- How would you improve the system for industrial deployment?

Conclusion: