College of Computer Studies

Laboratory Exercise No. 4

| Topic: | Ultrasonic sensor ESP32 | Week No. | 2 |
| --- | --- | --- | --- |
| Course Code: | CSST-105 | Term: | Inter Semester |
| Course Title: | Robotics | Academic Year: | 2025-2026 |

Ultrasonic Sensor with LCD Display for Distance Measurement

Objective

To understand and implement the principles of digital signal processing by interfacing an ultrasonic sensor with an LCD display to measure and display distance.

Materials Needed:

- ESP32 board or (Arduino board as an optional microcontroller)

- HC-SR04 Ultrasonic Sensor

- 16x2 LCD Display (with I2C module for simplicity) or OLED mini

- Breadboard and jumper wires

- Resistors (if not using I2C)

- Power supply (USB cable for Arduino)

- Computer with Arduino IDE installed

Background discussion

Ultrasonic sensors measure distance by sending out an ultrasonic pulse and timing how long it takes for the echo to return. This time is then converted into a distance measurement. The LCD display will show the measured distance in centimeters or inches.

This activity integrates principles of digital signal processing, basic electronics, and programming, providing a hands-on experience that reinforces theoretical knowledge.

Activity Steps:

Part 1: Wiring the Circuit

- Connect the Ultrasonic Sensor:

  - VCC pin of HC-SR04 to 5V on Arduino.

  - GND pin of HC-SR04 to GND on Arduino.

  - Trig pin to digital pin 9 on Arduino.

  - Echo pin to digital pin 10 on Arduino.

- Connect the LCD Display:

  - If using an I2C module:

    - VCC pin to 5V on Arduino.

    - GND pin to GND on Arduino.

    - SDA pin to A4 on Arduino (for Uno).

    - SCL pin to A5 on Arduino (for Uno).

  - If using parallel connection, connect according to the standard 16x2 LCD pinout.

Part 2: Programming the Arduino

- Open the Arduino IDE:

  - Create a new sketch.

- Include Libraries:

#include <LiquidCrystal_I2C.h>  // For I2C LCD

#define TRIG_PIN 9

#define ECHO_PIN 10

LiquidCrystal_I2C lcd(0x27, 16, 2); // Change 0x27 based on your I2C address

void setup() {

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

lcd.begin();

lcd.backlight();

lcd.print("Distance:");

}

void loop() {

long duration, distance;

// Clear the trigger

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Trigger the sensor

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read the echo

duration = pulseIn(ECHO_PIN, HIGH);

// Calculate the distance

distance = (duration * 0.034) / 2; // cm

// Display the distance on the LCD

lcd.setCursor(0, 1);

lcd.print(distance);

lcd.print(" cm  "); // Space to clear previous readings

delay(500); // Update every half second

}

Upload the Code:

  - Connect the Arduino to your computer and upload the sketch.

Part 3: Testing and Observations

- Testing the Setup:

  - Power the Arduino and observe the LCD.

  - Measure the distance using various objects placed at different distances from the sensor.

  - Confirm that the distance displayed changes accordingly.

- Collect Data:

  - Record the measured distances from different positions (e.g., 10 cm, 50 cm, 100 cm) and compare them to actual measurements using a ruler.

- Make Changes:

- Modify the code to switch between different units of measurement (e.g., cm to inches)?

- Add sound feedback (buzzer) that changes tone based on the distance measured.

Part 4:

Discussion Questions:

  - How does the ultrasonic sensor work?

  - What factors could affect the accuracy of distance measurements?

  - How does the LCD display work with the Arduino to show the measured data?

Summarized Activities

- Summarize what was learned about ultrasonic sensors and LCD interfacing.

- Discuss potential applications of this technology in robotics, automation, and other fields.